# Purpose:

The purpose of these Guidelines is to safeguard Buckeye Partners, L.P.’s (“Buckeye”) Stockholder(s) product integrity, as well as Buckeye personnel and assets.  These Marine Product Acceptance Guidelines cover the receipt and acceptance of petroleum products being delivered to Buckeye owned or operated terminals via Marine Ship or Barge (“Vessel”).  This includes both Fungible and Segregated Tank usage.

# Guidelines:

  - Vessel Acceptance and Scheduling

    - Prior to the Vessel’s arrival at a Buckeye Terminal or Notice of Readiness (NOR) being accepted, Customers are required to provide their respective Buckeye Terminal Scheduler and the receiving Buckeye Terminal Operations personnel (refer to Attachment A), a Certificate of Analysis (“COA”) from the vessel for each product which will be discharged at the respective Buckeye Terminal in accordance with these Guidelines.

    - If the test results are not available or samples cannot be obtained and tested prior to the marine vessel arriving at the Buckeye Terminal, then Buckeye at its sole discretion may permit the Customer to sample the marine vessel for testing upon mooring at the Buckeye Terminal contingent on berth availability and following  the requirements set forth in these guidelines.  Any expenses incurred by Buckeye as a result of any sampling and testing being performed at the Buckeye Terminal dock will be reimbursed to Buckeye by the Customer.  If necessary, Buckeye reserves the right to have the marine vessel vacate the Terminal berth while waiting for test results.

### Buckeye Terminal Operations personnel review all COA and vessel results against the Marine Terminal Product Receipt Specifications Tables or Contractual specification (if listed).  If the product(s) meets the Buckeye or Contractual Specifications, the vessel is approved to discharge.

    - If the product does not meet Buckeye or Contractual specifications (if listed), the respective Terminal Operations personnel will contact the Terminal Scheduler and the Terminal M&QC Specialist for further instructions.  Buckeye reserves the right to reject any product that does not meet Buckeye or Contractual specifications (if listed).

## Test Methods

### Buckeye will ONLY accept ASTM Test results (unless otherwise indicated in the slates) from a certified lab.

## Acceptable Certificates of Analysis Sampling and Test Results

### U.S. Domestically Sourced Vessels - Non-Buckeye Loading Terminals

#### Discharging into Fungible or Segregated Tank Storage

##### A full slate COA must be provided for each delivering vessel’s product(s) being discharged at a Buckeye Terminal.  The COA must report and meet the specifications listed in the respective Customer Storage Contract.  If no specifications are listed in the contract, Full Slate COA results as listed in the Marine Terminal Product Receipt Specifications – Table I (“Table I”) must be provided for each product being discharged at Buckeye Terminals.

Samples from the delivering vessel may be volumetrically composited from each applicable compartment or tested individually by compartment.  If the vessel contains or is suspected of containing H2S, H2S results must be reported for each compartment being discharged.

Or

##### A full slate COA reported off the load port tank unless the load port tank is active at time of loading.  The COA must report and meet the specifications listed in the respective Customer Storage Contract.  If no specifications are listed in the contract, Full Slate COA results as listed in Table I must be provided for each product being discharged at Buckeye Terminals.

##### If the Full Slate COA is based off of the load port tank, the customer must also have the delivering vessel tested in accordance with Marine Terminal Product Receipt Specifications – Table II (“Table II”) requirements prior to berthing or NOR acceptance.

### U.S. Domestically Sourced Vessels - Buckeye Loading Terminals

#### Discharging into Fungible Tank Storage

##### A full slate COA must be provided for each delivering vessel’s product(s) being discharged at a Buckeye Terminal.  The COA must report and meet the specifications listed in the respective Customer Storage Contract.  If no specifications are listed in the contract, Full Slate COA results as listed in the Table I must be provided for each product being discharged at Buckeye Terminals.

Samples from the delivering vessel may be volumetrically composited from each applicable compartment or tested individually by compartment.  If the vessel contains or is suspected of containing H2S, H2S results must be reported for each compartment being discharged.

Or

##### A full slate COA reported off the load port tank unless the load port tank is active at time of loading.  The COA must report and meet the specifications listed in the respective Customer Storage Contract.  If no specifications are listed in the contract, Full Slate COA results as listed in Table I must be provided for each product being discharged at Buckeye Terminals.

##### If the Full Slate COA is based off of the load port tank, the customer must also have the delivering vessel tested in accordance with Table II requirements prior to berthing or NOR acceptance.

#### Discharging into Segregated Tank Storage

##### Delivering vessel’s Table II test results are only required for each product being discharged at a Buckeye Terminal.

### Internationally Sourced Vessels

#### Discharging into Buckeye Terminal Fungible Tank Storage

##### A full slate COA must be provided from vessel samples taken once the vessel has arrived in U.S. waters (for Buckeye U.S. terminals) or local waters (for Buckeye International terminals).   The COA must report and meet the specifications listed in the respective Customer Storage Contract.  If no specifications are listed in the contract, Full Slate COA results as listed in the Table I must be provided for each product being discharged at Buckeye Terminals.

##### Samples from the delivering vessel may be volumetrically composited from each compartment which will be discharged or tested individually by compartment.  If the vessel contains or is suspected of containing H2S, H2S results must be reported for each compartment being discharged.

#### Discharging into Buckeye Terminal Segregated Tank Storage

##### A full slate COA must be provided for each product from each vessel compartment being discharged at a Buckeye Terminal.  The COA must report and meet the specifications listed in the respective Customer Storage Contract.  If no specifications are listed in the contract, Full Slate COA results as listed in the Table I must be provided for each product being discharged at Buckeye Terminals.

Samples from the delivering vessel may be volumetrically composited from each applicable compartment or tested individually by compartment.  If the vessel contains or is suspected of containing H2S, H2S results must be reported for each compartment being discharged.

Or

##### A full slate COA reported off the load port tank unless the load port tank is active at time of loading.  The COA must report and meet the specifications listed in the respective Customer Storage Contract.  If no specifications are listed in the contract, Full Slate COA results as listed in Table I must be provided for each product being discharged at Buckeye Terminals.

##### If the Full Slate COA is based off of the load port tank, the customer must also have the delivering vessel tested in accordance with Table II requirements prior to berthing or NOR acceptance.

### Receipts at Multiple Buckeye Terminals

#### For vessels discharging at multiple Buckeye Terminals, Table II testing does not need to be conducted at each port providing Table II tests results of those products being discharged have been received at previous ports and no other products have been loaded onto the vessel since the Table II testing was conducted.

## Terminal Exceptions

### Some terminals may impose more stringent requirements then those outlined above.  Please contact your terminal scheduler for details.

## Certificate of Analysis Time Durations

### Certificates of Analysis (CoA) shall include results from samples that have not surpassed the following time limits:

### Aviation Grades – CoA based on samples taken not more than 30 days before receipt by Buckeye.

- Other refined products (gasolines, diesel, heating oil, etc), including biodiesel and ethanol – CoA based on samples taken not more than 60 days before receipt by Buckeye.

Buckeye Marine Terminal Contact Listing

Attachment A (updated 08/01/2024)

| Terminal | Terminal Phone Number | M&QC Specialist | Scheduler |
| --- | --- | --- | --- |
| Albany, NY | 1-518-462-4237 | Anthony Trinkle | Piotr Bielunas |
| AlbanyAssetTeam@buckeye.com |  | atrinkle@buckeye.com | pbielunas@buckeye.com |
| AlbanyCustomerMovements@buckeye.com |  |  |  |
| Baltimore | 1-410-355-0705 | Anthony Trinkle | Ashley Pinter |
| BETQB@buckeye.com |  | atrinkle@buckeye.com | apinter@buckeye.com |
| BaltimoreCustomerMovements@buckeye.com |  |  |  |
| Bayonne | 1-201-437-1107 | Ranji Kuruvilla | Mike Neumann |
| BBTQY@buckeye.com |  | rkuruvilla@buckeye.com | aneumann@buckeye.com |
| BayonneCustomerMovements@buckeye.com |  |  |  |
| Buckeye Bahamas Hub | 1-242-350-2100 | Ranji Kuruvilla | Marine Scheduling Group |
| bbhassetteam@buckeye.com |  | rkuruvilla@buckeye.com | BBHopsplanning@buckeye.com |
| Charleston | 1-843-554-1581 | Mark Reedy | John Blackshear |
| BETQH@buckeye.com |  | mreedy@buckeye.com | jblackshear@buckeye.com |
| CharlestonCustomerMovements@buckeye.com |  |  |  |
| Chesapeake | 1-757-543-2061 | Anthony Trinkle | Ashley Pinter |
| BETQC@buckeye.com |  | atrinkle@buckeye.com | apinter@buckeye.com |
| ChesapeakeCustomerMovements@buckeye.com |  |  |  |
| Cincinnati | 1-513-941-1665 | Trevor Pizzo | Diana Koch |
| BETCXCincinnati@buckeye.com |  | tpizzo@buckeye.com | dkoch@buckeye.com |
| CincinnatiCustomerMovements@buckeye.com |  |  |  |
| Corpus Christi | 1-361-687-2820 | Mark Reedy | Gewyn Olson |
| buckeyetexashuboperators@buckeye.com |  | mreedy@buckeye.com | GOlson@buckeye.com |
| Ft Lauderdale | 1-954-523-3656 | Mark Reedy | Chuck Troutman |
| BETQD@buckeye.com |  | mreedy@buckeye.com | ctroutman@buckeye.com |
| FortLauderdaleCustomerMovements@buckeye.com |  |  |  |
| Groton | 1-860-445-7491 | Ranji Kuruvilla | Mike Neumann |
| BETQG@buckeye.com |  | rkuruvilla@buckeye.com | aneumann@buckeye.com |
| GrotonCustomerMovements@buckeye.com |  |  |  |
| Jacksonville | 1-904-757-4498 | Mark Reedy | Chuck Troutman |
| BETQJ@buckeye.com |  | mreedy@buckeye.com | ctroutman@buckeye.com |
| JacksonvilleCustomerMovements@buckeye.com |  |  |  |
| Louisville | 1-502-776-4671 | Trevor Pizzo | Aric Mills |
| BETVLLouisville@buckeye.com |  | tpizzo@buckeye.com | amills@buckeye.com |
| LouisvilleCustomerMovements@buckeye.com |  |  |  |
| Marrero, LA | 504-371-3900 | Mark Reedy | Brad Cortez |
| BMTUAMarrero@buckeye.com |  | mreedy@buckeye.com | bcortez@buckeye.com |
| Terminal | Terminal Phone Number | M&QC Specialist | Scheduler |
| Albany, NY | 1-518-462-4237 | Anthony Trinkle | Piotr Bielunas |
| AlbanyAssetTeam@buckeye.com |  | atrinkle@buckeye.com | pbielunas@buckeye.com |
| AlbanyCustomerMovements@buckeye.com |  |  |  |
| Baltimore | 1-410-355-0705 | Anthony Trinkle | Ashley Pinter |
| BETQB@buckeye.com |  | atrinkle@buckeye.com | apinter@buckeye.com |
| BaltimoreCustomerMovements@buckeye.com |  |  |  |
| Bayonne | 1-201-437-1107 | Ranji Kuruvilla | Mike Neumann |
| BBTQY@buckeye.com |  | rkuruvilla@buckeye.com | aneumann@buckeye.com |
| BayonneCustomerMovements@buckeye.com |  |  |  |
| Buckeye Bahamas Hub | 1-242-350-2100 | Ranji Kuruvilla | Marine Scheduling Group |
| bbhassetteam@buckeye.com |  | rkuruvilla@buckeye.com | BBHopsplanning@buckeye.com |
| Charleston | 1-843-554-1581 | Mark Reedy | John Blackshear |
| BETQH@buckeye.com |  | mreedy@buckeye.com | jblackshear@buckeye.com |
| CharlestonCustomerMovements@buckeye.com |  |  |  |
| Chesapeake | 1-757-543-2061 | Anthony Trinkle | Ashley Pinter |
| BETQC@buckeye.com |  | atrinkle@buckeye.com | apinter@buckeye.com |
| ChesapeakeCustomerMovements@buckeye.com |  |  |  |
| Cincinnati | 1-513-941-1665 | Trevor Pizzo | Diana Koch |
| BETCXCincinnati@buckeye.com |  | tpizzo@buckeye.com | dkoch@buckeye.com |
| CincinnatiCustomerMovements@buckeye.com |  |  |  |
| Corpus Christi | 1-361-687-2820 | Mark Reedy | Gewyn Olson |
| buckeyetexashuboperators@buckeye.com |  | mreedy@buckeye.com | GOlson@buckeye.com |
| Ft Lauderdale | 1-954-523-3656 | Mark Reedy | Chuck Troutman |
| BETQD@buckeye.com |  | mreedy@buckeye.com | ctroutman@buckeye.com |
| FortLauderdaleCustomerMovements@buckeye.com |  |  |  |
| Groton | 1-860-445-7491 | Ranji Kuruvilla | Mike Neumann |
| BETQG@buckeye.com |  | rkuruvilla@buckeye.com | aneumann@buckeye.com |
| GrotonCustomerMovements@buckeye.com |  |  |  |
| Jacksonville | 1-904-757-4498 | Mark Reedy | Chuck Troutman |
| BETQJ@buckeye.com |  | mreedy@buckeye.com | ctroutman@buckeye.com |
| JacksonvilleCustomerMovements@buckeye.com |  |  |  |
| Louisville | 1-502-776-4671 | Trevor Pizzo | Aric Mills |
| BETVLLouisville@buckeye.com |  | tpizzo@buckeye.com | amills@buckeye.com |
| LouisvilleCustomerMovements@buckeye.com |  |  |  |
| Marrero, LA | 504-371-3900 | Mark Reedy | Brad Cortez |
| BMTUAMarrero@buckeye.com |  | mreedy@buckeye.com | bcortez@buckeye.com |